MVM

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity fifo is

generic (depth:integer:=16);

Port ( data_in : in  STD_LOGIC_VECTOR (7 downto 0);

clk : in  STD_LOGIC;

enw : in  STD_LOGIC;

enr : in  STD_LOGIC;

reset : in  STD_LOGIC;

data_out : out  STD_LOGIC_VECTOR (7 downto 0);

fifo_empty : out  STD_LOGIC;

fifo_full : out  STD_LOGIC);

end fifo;

architecture fifo_arch of fifo is

type memory_type is array (0 to depth - 1) of STD_LOGIC_VECTOR(7 downto 0);

signal memory:memory_type:=(others=>(others=>'0'));

signal writeptr,readptr:integer:=0;

signal full,empty  : std_logic := '0';

begin

fifo_empty<=empty;

fifo_full<=full;

process(clk,reset)

variable num_elem:integer:=0;

begin

if(reset='1') then

memory<=(others=>(others=>'0'));

data_out<=(others=>'0');

empty<='1';

full<='0';

writeptr<=0;

readptr<=0;

num_elem:=0;

elsif(rising_edge(clk)) then

if(enr='1' and empty='0') then  --read

data_out<=memory(readptr);

readptr<=readptr+1;

num_elem:=num_elem-1;

end if;

if(enw='1' and full='0') then  --write

memory(writeptr)<=data_in;

writeptr<=writeptr+1;

num_elem:=num_elem+1;

end if;

if(readptr=depth-1) then

writeptr<=0;

end if;

if(writeptr=depth-1) then

readptr<=0;

end if;

if(num_elem = 0) then

empty <= '1';

else

empty <= '0';

end if;

if(num_elem = depth) then

full <= '1';

else

full <= '0';

end if;

end if;

end process;

end fifo_arch;

TESTBENCH

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.std_logic_ARITH.ALL;

ENTITY fifo_tb IS

END fifo_tb;

ARCHITECTURE behavior OF fifo_tb IS

--Inputs

signal data_in,data_out : std_logic_vector(7 downto 0) := (others => '0');

signal clk : std_logic := '0';

signal enw : std_logic := '0';

signal enr : std_logic := '0';

signal reset,empty,full : std_logic := '0';

signal i :integer:=0;

constant depth:integer:=16;

-- Clock period definitions

constant clk_period : time := 10 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: entity work.fifo generic map(depth=>depth) PORT MAP (

data_in => data_in,

clk => clk,

enw => enw,

enr => enr,

reset => reset,

data_out => data_out,

fifo_empty => empty,

fifo_full => full

);

-- Clock process definitions

clk_process :process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

-- Stimulus process

stim_proc: process

begin

reset<='1';

wait for clk_period;

reset<='0';

wait for clk_period*3;

enw<='1'; enr<='0';

for i in 1 to 10 loop

data_in<=conv_std_logic_vector(i,8);

wait for clk_period;

end loop;

enw<='0'; enr<='1';

wait for clk_period*4;

enw<='0'; enr<='0';

wait for clk_period;

for i in 11 to 20 loop

data_in<=conv_std_logic_vector(i,8);

wait for clk_period;

end loop;

enw<='0'; enr<='0';

wait for clk_period*10;

enw<='0'; enr<='1';

wait for clk_period*4;

enw<='0'; enr<='0';

wait for clk_period;

enw<='0'; enr<='1';

wait for clk_period*4;

enw<='0'; enr<='0';

wait for clk_period;

enw<='0'; enr<='1';

wait for clk_period*4;

enw<='0'; enr<='0';

wait for clk_period;

enw<='0'; enr<='1';

wait for clk_period*4;

enw<='0'; enr<='0';

wait;end process;END;