MVM

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity usr is

Port ( rst : in  STD_LOGIC;

clk : in  STD_LOGIC;

Pin : in  STD_LOGIC_VECTOR (3 downto 0);

Sin : in  STD_LOGIC;

mode : in  STD_LOGIC_VECTOR (1 downto 0);

Pout : out  STD_LOGIC_VECTOR (3 downto 0);

Sout : out  STD_LOGIC);

end usr;

architecture usr_arch of usr is

signal temp:STD_LOGIC_VECTOR(3 DOWNTO 0) := "0000";

begin

PROCESS(rst,clk,mode,Sin,Pin)

begin

if rst='1' then

Pout<="0000";

Sout<='0';

elsif FALLING_EDGE(clk) then

case mode is

when "00" =>

temp(3 downto 1)<= temp(2 downto 0);

temp(0)<= Sin;

Sout<=temp(3);

Pout<="0000";

when "01" =>

temp(3 downto 1)<= temp(2 downto 0);

temp(0)<= Sin;

Pout<=temp;

Sout<='0';

when "10" =>

temp<= Pin;

Sout<= temp(3);

Pout<="0000";

when others =>

Pout<=Pin;

Sout<='0';

end case;

end if;

end process;

end usr_arch;

TESTBENCH

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

ENTITY usr_tb IS

END usr_tb;

ARCHITECTURE behavior OF usr_tb IS

-- Component Declaration for the Unit Under Test (UUT)

COMPONENT usr

PORT(

rst : IN  std_logic;

clk : IN  std_logic;

Pin : IN  std_logic_vector(3 downto 0);

Sin : IN  std_logic;

mode : IN  std_logic_vector(1 downto 0);

Pout : OUT  std_logic_vector(3 downto 0);

Sout : OUT  std_logic

);

END COMPONENT;

--Inputs

signal rst : std_logic := '0';

signal clk : std_logic := '1';

signal Pin : std_logic_vector(3 downto 0) := "1010";

signal Sin : std_logic := '0';

signal mode : std_logic_vector(1 downto 0) := (others => '0');

--Outputs

signal Pout : std_logic_vector(3 downto 0);

signal Sout : std_logic;

-- Clock period definitions

constant clk_period : time := 10 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: usr PORT MAP (

rst => rst,

clk => clk,

Pin => Pin,

Sin => Sin,

mode => mode,

Pout => Pout,

Sout => Sout

);

-- Clock process definitions

clk_process :process

begin

clk <= NOT(clk);

wait for clk_period/2;

end process;

-- Stimulus process

stim_proc_mode: process

begin

mode<="00";

wait for 80 ns;

mode<="01";

wait for 50 ns;

mode<="10";

wait for 50 ns;

mode<="11";

wait for 20 ns;

end process;

stim_proc_Sin: process

begin

wait for 10 ns;

Sin<='1';

wait for 10 ns;

Sin<='0';

wait for 10 ns;

Sin<='1';

wait for 10 ns;

Sin<='0';

wait for 10 ns;

Sin<='0';

wait for 40 ns;

Sin<='1';

wait for 10 ns;

Sin<='0';

wait for 10 ns;

Sin<='1';

wait for 10 ns;

Sin<='0';

wait for 10 ns;

Sin<='0';

wait;

end process;

stim_proc_rst: process

begin

wait for 200 ns;

rst<='1';

wait for 5 ns;

rst<='0';

wait;

end process;

END;